Can the vision camera recognise the O-O move, castling short.

Yes, your current vision system can recognize the O-O (Short Castling) and O-O-O (Long Castling) moves without any extra code changes.

Here is why it works and the one rule you must follow:

1. How the Logic Works

In the python-chess library (which runs your game logic), castling is technically encoded as "The King moves two squares."

- White Short Castle (O-O): Defined as e1 $\to$ g1.

- White Long Castle (O-O-O): Defined as e1 $\to$ c1.

Your current vision loop (detect_move) looks for changes on the board.

- You physically move the King from e1 to g1.

- You physically move the Rook from h1 to f1.

- The camera detects changes at 4 squares: ['e1', 'g1', 'h1', 'f1'].

- The code checks the legal moves: "Is e1 $\to$ g1 a valid move?"

- Yes. It sees that e1 changed and g1 changed. It accepts the move e1g1.

- The library automatically teleports the Rook internally to match the castling rule.

2. The Critical Rule for You (The Human)

Because the camera only needs to see the King move to register the command, you MUST physically move the Rook yourself.

If you move only the King:

- The Code will think: "Castling complete. Rook is now at f1."

- The Reality: Rook is still at h1.

- The Crash: Later, if the robot tries to move that Rook from f1, it will grab thin air. Or if it tries to move to h1, it might crash into the Rook you forgot to move.

Summary

- Recognition: Automatic. (It triggers off the King's start/end points).

- Action: Move BOTH the King and Rook clearly before you let the camera "lock in" the move.

Does the robot arm move twice or 3 times for O-O (Short Castling) and O-O-O (Long Castling)

This is a great question. In standard chess robotics, Castling is treated as 2 Separate Moves performed back-to-back.

- Move 1 (The King): E1 $\to$ G1 (Short) or E1 $\to$ C1 (Long).

- Move 2 (The Rook): H1 $\to$ F1 (Short) or A1 $\to$ D1 (Long).

⚠️ The "Missing Rook" Bug

In the v8.5 code I just gave you, there is a small logic gap: It only sees the King's move.

If you run the current code, the robot will move the King to G1, but it will forget to move the Rook, leaving your board illegal physically (even though the computer knows the Rook moved).

The Fix: Add the "Castling Check"

You need to add a specific check for board.is_castling(result.move) inside the game loop. This ensures the robot performs the second move (the Rook) immediately after the King.

Here is the corrected Logic Block for Section 4 (Vision & Game Loop). Replace the middle part of the loop with this:

Python

# ... (Inside the 'if board.turn == is_white:' block) ...

m_str = result.move.uci()

sq_src, sq_dst = m_str[0:2], m_str[2:4]

x1, y1, _ = mapper.get_coords(sq_src)

x2, y2, _ = mapper.get_coords(sq_dst)

# --- 1. HANDLE CAPTURE (3 Steps: Pick Enemy -> Trash -> Move Own) ---

if board.is_capture(result.move):

print(f"DEBUG: Capture Sequence {sq_dst}")

# A. Remove Enemy

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

land(sq_dst, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_CLOSED_ANGLE)

smart_move(TRASH_X, TRASH_Y, TRASH_Z, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

# B. Move Own Piece

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)

land(sq_src, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE)

land(sq_dst, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

# --- 2. HANDLE CASTLING (2 Steps: Move King -> Move Rook) ---

elif board.is_castling(result.move):

print(f"DEBUG: Castling Sequence {m_str}")

# A. Move King (Standard)

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)

land(sq_src, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE)

land(sq_dst, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

# B. Move Rook (Calculated based on King's move)

rook_src, rook_dst = "", ""

# White Short (e1g1) -> Rook h1f1

if sq_dst == "g1": rook_src, rook_dst = "h1", "f1"

# White Long (e1c1) -> Rook a1d1

elif sq_dst == "c1": rook_src, rook_dst = "a1", "d1"

# Black Short (e8g8) -> Rook h8f8

elif sq_dst == "g8": rook_src, rook_dst = "h8", "f8"

# Black Long (e8c8) -> Rook a8d8

elif sq_dst == "c8": rook_src, rook_dst = "a8", "d8"

# Execute Rook Move

rx1, ry1, _ = mapper.get_coords(rook_src)

rx2, ry2, _ = mapper.get_coords(rook_dst)

time.sleep(0.5) # Pause between King and Rook

smart_move(rx1, ry1, Z_HOVER, GRIP_OPEN_ANGLE)

land(rook_src, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(rx1, ry1, Z_HOVER, GRIP_CLOSED_ANGLE)

land(rook_dst, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

smart_move(rx2, ry2, Z_HOVER, GRIP_OPEN_ANGLE)

# --- 3. STANDARD MOVE (1 Step) ---

else:

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)

land(sq_src, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_CLOSED_ANGLE)

land(sq_dst, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

# --- Finish Turn ---

robot.home()

# ...

Summary of Robot Moves

| Move Type | Robot Actions | Description |
| --- | --- | --- |
| Standard | 1 | Pick A, Place B. |
| Capture | 2 | Pick Enemy -> Trash. Pick Own -> Place. |
| Castling | 2 | Pick King -> Place. Pick Rook -> Place. |

Explain to me how to adjut these z-parameters: TILT_CURVE = 0.0035

TILT_SLOPE = 0.04

TILT_INTERCEPT = -3.0

MAX_TILT_DROP = -10.0

These four parameters control the "Invisible Floor" that your robot calculates.

Because of gravity, the weight of the servos, and loose gears, the robot arm physically sags the further out it reaches. The software thinks it is moving in a straight line, but in reality, the arm is curving downward.

These parameters create a mathematical "hill" to counteract that physical "sag."

Here is how to tune them, in this specific order:

1. TILT_INTERCEPT (The "Elevator")

Current: -3.0

What it does: Moves the entire board height up or down.

- Symptom: The robot is too high (or too low) at A1 (Close to base) AND H8 (Far away).

- How to tune:

  - Go to square A1 (closest to robot).

  - If the gripper hits the board: Increase (e.g., -2.0, -1.0).

  - If the gripper is grabbing air above the piece: Decrease (e.g., -4.0, -5.0).

  - Get this perfect at A1 before touching anything else.

2. TILT_SLOPE (The "Ramp")

Current: 0.04

What it does: Tilts the floor angle. It adds height as the arm reaches further out.

- Symptom: A1 is perfect, but H8 (Far corner) is crashing into the board.

- How to tune:

  - Check A1 (should be perfect from step 1).

  - Go to H8.

  - If it crashes into the board at H8: Increase (e.g., 0.05, 0.06). This lifts the far end up.

  - If it is too high at H8: Decrease (e.g., 0.03, 0.02).

3. TILT_CURVE (The "Sag Fixer")

Current: 0.0035

What it does: Adds extra lift only at the very extreme distances. This fights the "bending" of the plastic arm at full extension.

- Symptom: A1 is good, E4 (middle) is good, but H8 (Far) still hits the board even after adjusting the slope.

- How to tune:

  - If the arm seems to "nosedive" suddenly at the very far edge: Increase slightly (e.g., 0.0040).

  - If the arm lifts up too high at the far edge (flying over the pieces): Decrease (e.g., 0.0020 or 0).

4. MAX_TILT_DROP (The Safety Floor)

Current: -10.0

What it does: This is a safety limit. It tells the robot: "Never calculate a Z-height lower than -10cm, no matter what the math says."

- How to tune:

  - Leave this alone unless your robot is trying to drill through the table. -10.0 is usually safe.

The "Quick Fix" Cheat Sheet

| Problem | Solution |
| --- | --- |
| Everywhere is too low (hitting board at A1, E4, H8) | Increase TILT_INTERCEPT (e.g. -3.0 $\to$ -2.0) |
| Everywhere is too high (missing pieces) | Decrease TILT_INTERCEPT (e.g. -3.0 $\to$ -4.0) |
| A1 is perfect, but H8 is too low (crashing far away) | Increase TILT_SLOPE (e.g. 0.04 $\to$ 0.05) |
| A1 is perfect, but H8 is too high (air grab far away) | Decrease TILT_SLOPE (e.g. 0.04 $\to$ 0.03) |

The radio button robot is white. if i do not check, means robot is black?

Yes, exactly.

- Checked ($\checkmark$): Robot is White.

  - The robot will make the first move immediately when you click "START GAME".

- Unchecked (Empty): Robot is Black.

  - The robot will wait. It will do nothing until the camera detects that YOU (White) have made the first move.

⚠️ Important Note for "Robot as Black"

When the robot is Black, the code looks for a file named vision_black.json.

- If you have not calibrated vision specifically for Black, the robot might not see the board correctly.

- Quick Fix: If you calibrated successfully as White and the camera hasn't moved, the grid is likely the same. You can just copy your vision_white.json and rename it to vision_black.json.

My physical arrangement is A1-H1 at the base of robot will put robot as white. Is that correrct?

Yes, that is 100% correct.

Here is the simple logic:

- Rank 1 (A1-H1) is where the White Pieces start.

- Rank 8 (A8-H8) is where the Black Pieces start.

Since your robot is sitting at the A1-H1 edge, it is physically sitting on the "White side" of the table.

- Best setup: The robot plays White (moves pieces close to it).

- Hard setup: If the robot played Black here, it would have to reach all the way across the table (to Rank 8) just to make its first move.

You are set up perfectly. Check the "Robot is White" box and click Start.

If I do not click Robot is white, it is playing black. I am playing white, which means we are playing on the wrong side of the chessboard if the board is not physically turned around?

Yes, you are absolutely correct.

If you (White) and the Robot (Black) are both physically located at the Rank 1 (Base) side of the table:

- You are fighting for space: You need to sit near the robot to move your White pieces (Ranks 1 & 2).

- The Robot is reaching dangerously far: The robot has to reach all the way across the board (Ranks 7 & 8) to move its own Black pieces.

- "Crossing Swords": To make a move, you have to reach under the robot, and the robot has to reach over your pieces.

The "Turn Around" Trap (Warning)

You might think: "I'll just turn the board 180 degrees so Black is near the robot."

DO NOT DO THIS without Re-Calibrating.

The robot is blind. It relies on your calibration.json file, which currently says:

"The corner closest to my base is A1."

If you spin the board 180°:

- The corner closest to the base becomes H8.

- The robot does not know this. It will think H8 is A1.

- It will try to play the "White Opening" by grabbing your Black pieces!

Recommendation

For your current physical setup (Robot at A1/H1):

- Best: Robot plays White. (It handles the pieces closest to it).

- If you MUST play Black: You (the human) should sit on the opposite side of the table (near Rank 8) and let the robot play White.

Stick to "Robot is White" for the safest and most reliable experience.

I think the best solution is to create a second calibration and overwrite json for Robot plays black, with Board physically turn around. So when I I choose Robot play black, it loads the second set of json files instead of the first set.

You are absolutely right. That is the cleanest and most professional solution.

Instead of forcing one set of math to fit two different physical setups, we will simply create two separate "Profiles":

- Profile A (White): Board normal. Robot uses calibration_white.json.

- Profile B (Black): Board rotated 180°. Robot uses calibration_black.json.

This allows you to have completely different Pins and Corners for each side.

Instructions for v8.5

- Select Profile: Go to "Jog & Calib". You will see a new switch: [ Edit WHITE Profile ] / [ Edit BLACK Profile ].

- Calibrate Black:

  - Turn the board physically.

  - Select "Edit BLACK Profile".

  - Jog to the corner that is now at A1's location (physically far away). Click Save A1.

  - Jog to the corner closest to the robot (which is now H8). Click Save H8.

  - Save Calibration File. (This will now save to calibration_black.json).

- Play: In the "Vision Game" tab, when you select "Robot is BLACK", the system will automatically load the Black files.

wait, the override function, does it also need both profiles?

- The "Physical Move" Problem:

  - When Robot is White, square E4 is physically close to the robot (about 20cm away).

  - When Robot is Black (and board is rotated), square E4 is physically far away (about 50cm away).

- The Result:

  - If you used the same Pin for both, the robot would try to go to the "Short E4" while playing Black, and it would drop the piece on the wrong side of the board.

The Good News: The code I gave you (v8.5) handles this automatically.

- When you select "Edit WHITE Profile", clicking Pin saves to overrides_white.json.

- When you select "Edit BLACK Profile", clicking Pin saves to overrides_black.json.

You do not need to do anything extra. Just make sure you select the correct Edit Profile radio button before you click "Pin".

It seems that the z-setting for slope only applies for A8 and H8. It does not seem to work from B8 to G8.

The Z-parameters (Slope/Curve) are calculated based on Distance from the Robot Base, not "Rank."

This creates a "Geometry Trap" on the back rank (Row 8):

- A8 & H8 (Corners): These are physically farther away (diagonal distance). The math gives them More Lift.

- D8 & E8 (Middle): These are physically closer than the corners. The math gives them Less Lift.

If your robot arm sags heavily at the far end, D8 and E8 are not getting enough lift because the math thinks they are "closer and safer" than A8.

The Solution: Use the Pin Tool

Since the math cannot fix "Middle Sag" without ruining the corners, you must use the Override (Pin) function for the middle squares of the back rank.

- Select the correct profile (e.g., "Edit WHITE Profile" if playing White).

- Jog to B8. Adjust height perfectly. Click Override (Pin XYZ).

- Jog to C8. Adjust height perfectly. Click Override (Pin XYZ).

- Repeat for D8, E8, F8, G8.

Do not pin A8 or H8 (let the Calibration handle the corners). Pinning the middle squares creates a "bridge" of safety across the back rank.

Does it cover O-O-O (Long Castling) move

Yes, the logic I provided in the snippet does cover O-O-O (Long Castling) for both White and Black.

Here is exactly how the code handles it:

- White Long Castling (O-O-O):

  - Trigger: The King moves to c1.

  - Action 1: Robot moves King from e1 to c1.

  - Action 2: Robot automatically calculates that for c1, the Rook must move from a1 to d1.

- Black Long Castling:

  - Trigger: The King moves to c8.

  - Action 1: Robot moves King from e8 to c8.

  - Action 2: Robot automatically moves Rook from a8 to d8.

The Complete Code (v8.6)

Since sticking the snippet into the previous file can be tricky, here is the complete, final script (v8.6). This version includes Dual Profiles, Safe Capture, Pinning, and Full Castling (Short & Long) support built-in.

You can simply replace your entire file with this one.

Python

"""

==================================================================================

CHESS ROBOT MONOLITH v8.6 (Full Castling & Dual Profiles)

==================================================================================

FEATURES:

1.  CASTLING: Handles O-O (Short) and O-O-O (Long) by moving both King and Rook.

2.  DUAL PROFILES: Separate calibration for White vs Black sides.

3.  SAFE CAPTURE: Trash can logic for captured pieces.

4.  SYNCED STATE: Prevents simulation errors.

"""

import tkinter as tk

from tkinter import ttk

import cv2

import numpy as np

import json

import os

import time

import threading

import math

import serial

import chess

import chess.engine

from tkinter import messagebox

# ==================================================================================

# SECTION 1: CONFIGURATION

# ==================================================================================

STOCKFISH_PATH = r"stockfish/stockfish-windows-2022-x86-64-avx2.exe"

CAMERA_INDEX = 0

# --- ROBOT DIMENSIONS (cm) ---

L_BASE_H = 7.0

L_SHOULDER = 18.0

L_ELBOW = 20.0

L_WRIST = 17.0

# --- OFFSETS ---

OFFSET_BASE = 16.0

OFFSET_SHOULDER = 4.0

OFFSET_ELBOW = 0.0

OFFSET_WRIST = 0.0

# --- HEIGHT STRATEGY ---

Z_TRAVEL = 8.0

Z_GRIP = 2.0

Z_HOVER = 12.0

# --- TRASH CAN (Shared) ---

TRASH_X = -15.0

TRASH_Y = 15.0

TRASH_Z = 5.0

# --- GRIPPER ---

GRIP_OPEN_ANGLE = 60

GRIP_CLOSED_ANGLE = 105

# --- TILT CORRECTION ---

TILT_CURVE = 0.0035

TILT_SLOPE = 0.04

TILT_INTERCEPT = -3.0

MAX_TILT_DROP = -10.0

# --- SAFETY LIMITS ---

SHLD_MIN = 0; SHLD_MAX = 175

ELBW_MIN = 0; ELBW_MAX = 180

WRST_MIN = 0; WRST_MAX = 180

MIN_SAFE_SHOULDER_ANGLE = 55.0

# ==================================================================================

# SECTION 2: HARDWARE DRIVER

# ==================================================================================

class ChessRobotArm:

def __init__(self, port='COM8'):

self.connected = False

try:

self.ser = serial.Serial(port, 9600, timeout=1)

print(f"[HW] Connected on {port}")

self.connected = True

time.sleep(5)

except Exception as e:

print(f"[HW] Simulation Mode: {e}")

self.gripper_state = self.deg_to_us(GRIP_OPEN_ANGLE)

def deg_to_us(self, deg):

d = max(0, min(180, deg))

return int((d / 180.0) * (2400 - 400) + 400)

def send_cmd(self, b, s, e, w, g):

if not self.connected: return

b_final = max(0, min(180, b + OFFSET_BASE))

s_final = max(SHLD_MIN, min(SHLD_MAX, s + OFFSET_SHOULDER))

e_final = max(ELBW_MIN, min(ELBW_MAX, e + OFFSET_ELBOW))

w_final = max(WRST_MIN, min(WRST_MAX, w + OFFSET_WRIST))

msg = f"{self.deg_to_us(b_final)},{self.deg_to_us(s_final)},{self.deg_to_us(e_final)},{self.deg_to_us(w_final)},{self.deg_to_us(g)}\n"

self.ser.write(msg.encode())

while True:

if self.ser.in_waiting and self.ser.readline().decode().strip() == "done": break

def park(self):

if not self.connected: return

print("[HW] Parking...")

msg = f"1550,1400,1289,1289,{self.deg_to_us(GRIP_CLOSED_ANGLE)}\n"

self.ser.write(msg.encode())

while True:

if self.ser.in_waiting and self.ser.readline().decode().strip() == "done": break

def inverse_kinematics(self, x, y, z_height=Z_TRAVEL):

base_angle = math.atan2(y, x) * 180 / math.pi

if base_angle < 0: base_angle += 360

r_total = math.sqrt(x**2 + y**2)

z_final = z_height

best_sol = None

for pitch in range(-90, 5, 5):

rad_p = math.radians(pitch)

dr = L_WRIST * math.cos(rad_p); dz = L_WRIST * math.sin(rad_p)

r_w = r_total - dr; z_w = (z_final - dz) - L_BASE_H

h = math.sqrt(r_w**2 + z_w**2)

if h > (L_SHOULDER + L_ELBOW): continue

try:

alpha = math.acos((L_SHOULDER**2 + h**2 - L_ELBOW**2) / (2 * L_SHOULDER * h))

beta = math.acos((L_SHOULDER**2 + L_ELBOW**2 - h**2) / (2 * L_SHOULDER * L_ELBOW))

except ValueError: continue

theta = math.atan2(z_w, r_w)

s = (theta + alpha) * 180 / math.pi; e = beta * 180 / math.pi

w = (s - (180 - e)) - pitch

if s > SHLD_MAX: continue

if s >= MIN_SAFE_SHOULDER_ANGLE: return base_angle, s, e, w

if best_sol is None or s > best_sol[1]: best_sol = (base_angle, s, e, w)

return best_sol

def move_to_xyz(self, x, y, z=Z_TRAVEL):

angles = self.inverse_kinematics(x, y, z_height=z)

if angles:

self.send_cmd(*angles, self.gripper_state)

return True

return False

def home(self): self.send_cmd(90, 90, 180, 18, GRIP_OPEN_ANGLE)

# ==================================================================================

# SECTION 3: MATH HELPERS

# ==================================================================================

def get_xy_from_angles(b, s, e, w):

rb, rs = math.radians(b), math.radians(s)

ge = s - (180 - e); re = math.radians(ge)

gw = ge - w; rw = math.radians(gw)

r = (L_SHOULDER*math.cos(rs)) + (L_ELBOW*math.cos(re)) + (L_WRIST*math.cos(rw))

z = L_BASE_H + (L_SHOULDER*math.sin(rs)) + (L_ELBOW*math.sin(re)) + (L_WRIST*math.sin(rw))

return r * math.cos(rb), r * math.sin(rb), z

class BoardMapper:

def __init__(self, is_white_profile=True):

suffix = "white" if is_white_profile else "black"

self.cal_file = f"calibration_{suffix}.json"

self.ovr_file = f"overrides_{suffix}.json"

print(f"[MAPPER] Loading Profile: {suffix.upper()}")

self.ovr = {}

if os.path.exists(self.ovr_file):

try: self.ovr = json.load(open(self.ovr_file, "r"))

except: pass

self.phys_cal = {}

if os.path.exists(self.cal_file):

with open(self.cal_file, "r") as f: self.phys_cal = json.load(f)

else:

print(f"[WARN] No {self.cal_file} found. Please calibrate for {suffix}.")

def get_coords(self, sq):

sq = sq.strip().lower()

if sq in self.ovr:

val = self.ovr[sq]

if len(val) >= 3: return val[0], val[1], val[2]

if sq in self.phys_cal:

x, y = self.phys_cal[sq]

dist = math.sqrt(x**2 + y**2)

z = max(MAX_TILT_DROP, (dist**2 * TILT_CURVE) + (dist * TILT_SLOPE) + TILT_INTERCEPT) + Z_GRIP

return x, y, z

col = ord(sq[0]) - ord('a'); row = int(sq[1]) - 1

xa1, ya1 = self.phys_cal.get("a1", [0,0]); xa8, ya8 = self.phys_cal.get("a8", [0,1])

xh1, yh1 = self.phys_cal.get("h1", [1,0]); xh8, yh8 = self.phys_cal.get("h8", [1,1])

u = col / 7.0; v = row / 7.0

x = (1-u)*(1-v)*xa1 + u*(1-v)*xh1 + u*v*xh8 + (1-u)*v*xa8

y = (1-u)*(1-v)*ya1 + u*(1-v)*yh1 + u*v*yh8 + (1-u)*v*ya8

dist = math.sqrt(x**2 + y**2)

z = max(MAX_TILT_DROP, (dist**2 * TILT_CURVE) + (dist * TILT_SLOPE) + TILT_INTERCEPT) + Z_GRIP

# print(f"DEBUG: {sq} -> Interpolated -> CM({x:.1f}, {y:.1f})")

return x, y, z

# ==================================================================================

# SECTION 4: VISION & GAME LOOP

# ==================================================================================

def interpolate_grid(tl, tr, br, bl):

dict_sq = {}

files = "abcdefgh"; ranks = "87654321"

src = np.array([tl, tr, br, bl], dtype="float32")

dst = np.array([[0,0], [800,0], [800,800], [0,800]], dtype="float32")

M = cv2.getPerspectiveTransform(dst, src)

step = 100.0

for r_idx, rank in enumerate(ranks):

for f_idx, file in enumerate(files):

x1, y1 = f_idx * step, r_idx * step

pts = np.array([[[x1,y1]], [[x1+step,y1]], [[x1+step,y1+step]], [[x1,y1+step]]], dtype="float32")

cam_pts = cv2.perspectiveTransform(pts, M)

dict_sq[file + rank] = [[int(c[0][0]), int(c[0][1])] for c in cam_pts]

return dict_sq

def detect_move(frame1, frame2, zones):

diff = cv2.absdiff(frame1, frame2)

gray = cv2.cvtColor(diff, cv2.COLOR_BGR2GRAY)

_, thresh = cv2.threshold(gray, 40, 255, cv2.THRESH_BINARY)

changed = []

for sq, poly in zones.items():

mask = np.zeros(thresh.shape, dtype=np.uint8)

cv2.fillPoly(mask, [np.array(poly, dtype=np.int32)], 255)

score = cv2.countNonZero(cv2.bitwise_and(thresh, thresh, mask=mask))

if score > 150: changed.append((sq, score))

changed.sort(key=lambda x: x[1], reverse=True)

return [x[0] for x in changed]

def run_vision_game(robot, is_white, app):

# AUTO-LOAD CORRECT PROFILE

mapper = BoardMapper(is_white_profile=is_white)

robot_color = "WHITE" if is_white else "BLACK"

v_file = f"vision_{robot_color.lower()}.json"

if not os.path.exists(v_file): return print(f"CRITICAL: No vision file {v_file}")

with open(v_file, 'r') as f: v_zones = json.load(f)

print(f"--- GAME STARTING ({robot_color}) ---")

engine = chess.engine.SimpleEngine.popen_uci(STOCKFISH_PATH)

cap = cv2.VideoCapture(CAMERA_INDEX, cv2.CAP_DSHOW)

time.sleep(1)

board = chess.Board()

cur_b, cur_s, cur_e, cur_w = 90, 90, 180, 18

def smart_move(x, y, z, g):

nonlocal cur_b, cur_s, cur_e, cur_w

angles = robot.inverse_kinematics(x, y, z_height=z)

if angles:

robot.send_cmd(angles[0], angles[1], angles[2], angles[3], g)

cur_b, cur_s, cur_e, cur_w = angles

robot.gripper_state = g

time.sleep(0.8)

else: print(f"WARN: Cannot reach {x:.1f}, {y:.1f}, {z:.1f}")

def raw_move(b, s, e, w, g):

nonlocal cur_b, cur_s, cur_e, cur_w

if app.stop_requested: return

robot.send_cmd(b, s, e, w, g)

cur_b, cur_s, cur_e, cur_w = b, s, e, w

robot.gripper_state = g

time.sleep(0.8)

def land(sq, grip_angle):

if app.stop_requested: return

r = mapper.get_coords(sq)

z = r[2] if len(r) > 2 and r[2] else Z_GRIP + max(MAX_TILT_DROP, (math.sqrt(r[0]**2 + r[1]**2)**2 * TILT_CURVE) + (math.sqrt(r[0]**2 + r[1]**2) * TILT_SLOPE) + TILT_INTERCEPT)

angles = robot.inverse_kinematics(r[0], r[1], z_height=z)

if angles: raw_move(angles[0], angles[1], angles[2], angles[3], grip_angle)

ret, frame_base = cap.read()

while not ret: time.sleep(0.5); ret, frame_base = cap.read()

while not board.is_game_over():

if app.stop_requested: break

if (board.turn == is_white):

print(f"[{robot_color}] Thinking...")

result = engine.play(board, chess.engine.Limit(time=0.8))

m_str = result.move.uci()

sq_src, sq_dst = m_str[0:2], m_str[2:4]

x1, y1, _ = mapper.get_coords(sq_src)

x2, y2, _ = mapper.get_coords(sq_dst)

# --- 1. CAPTURE ---

if board.is_capture(result.move):

print(f"DEBUG: Capturing {sq_dst}")

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

land(sq_dst, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_CLOSED_ANGLE)

smart_move(TRASH_X, TRASH_Y, TRASH_Z, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

time.sleep(0.5)

# After Trash, proceed to move own piece:

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)

land(sq_src, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE)

land(sq_dst, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

# --- 2. CASTLING (O-O and O-O-O) ---

elif board.is_castling(result.move):

print(f"DEBUG: Castling {m_str}")

# A. Move King

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)

land(sq_src, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE)

land(sq_dst, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

# B. Move Rook

rook_src, rook_dst = "", ""

if sq_dst == "g1": rook_src, rook_dst = "h1", "f1" # White Short

elif sq_dst == "c1": rook_src, rook_dst = "a1", "d1" # White Long

elif sq_dst == "g8": rook_src, rook_dst = "h8", "f8" # Black Short

elif sq_dst == "c8": rook_src, rook_dst = "a8", "d8" # Black Long

rx1, ry1, _ = mapper.get_coords(rook_src)

rx2, ry2, _ = mapper.get_coords(rook_dst)

time.sleep(0.5)

smart_move(rx1, ry1, Z_HOVER, GRIP_OPEN_ANGLE)

land(rook_src, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(rx1, ry1, Z_HOVER, GRIP_CLOSED_ANGLE)

land(rook_dst, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

smart_move(rx2, ry2, Z_HOVER, GRIP_OPEN_ANGLE)

# --- 3. STANDARD MOVE ---

else:

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)

land(sq_src, GRIP_OPEN_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_CLOSED_ANGLE)

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_CLOSED_ANGLE)

land(sq_dst, GRIP_CLOSED_ANGLE)

raw_move(cur_b, cur_s, cur_e, cur_w, GRIP_OPEN_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

robot.home(); cur_b, cur_s, cur_e, cur_w = 90, 90, 180, 18

board.push(result.move)

time.sleep(2.0); ret, frame_base = cap.read()

while not ret: time.sleep(0.5); ret, frame_base = cap.read()

else:

print(f"\n[HUMAN] Your Turn...")

user_move = None

while not user_move:

if app.stop_requested: break

ret, frame = cap.read()

if not ret: time.sleep(0.2); continue

vis = frame.copy()

for sq, poly in v_zones.items(): cv2.polylines(vis, [np.array(poly, np.int32)], True, (0,255,0), 1)

cv2.imshow("Chess Vision", vis)

if cv2.waitKey(1) == 32:

changes = detect_move(frame_base, frame, v_zones)

for m in board.legal_moves:

if chess.square_name(m.from_square) in changes and chess.square_name(m.to_square) in changes:

user_move = m; break

if app.stop_requested: break

board.push(user_move); frame_base = frame.copy()

cap.release(); cv2.destroyAllWindows(); engine.quit()

print("Game Stopped."); robot.park()

# ==================================================================================

# SECTION 5: GUI

# ==================================================================================

class RobotGUI:

def __init__(self, root):

self.root = root; self.root.title("Chess Robot Monolith v8.6 (Full Castling)")

self.root.geometry("600x850")

self.robot = ChessRobotArm()

self.edit_profile_var = tk.BooleanVar(value=True) # Needs to be defined before Mapper

self.mapper = BoardMapper(is_white_profile=True)

self.setup_ui()

self.b, self.s, self.e, self.w, self.g = 90, 90, 180, 18, GRIP_OPEN_ANGLE

self.load_calib_ui()

self.vis_running = False; self.vis_pts = {}; self.last_click = None

def log(self, msg): self.txt.insert(tk.END, f"[{time.strftime('%H:%M:%S')}] {msg}\n"); self.txt.see(tk.END)

def setup_ui(self):

tabs = ttk.Notebook(self.root); tabs.pack(expand=1, fill="both")

t_jog = ttk.Frame(tabs); tabs.add(t_jog, text='1. Jog & Calib')

t_test = ttk.Frame(tabs); tabs.add(t_test, text='2. Test & Fix')

t_game = ttk.Frame(tabs); tabs.add(t_game, text='3. Vision Game')

# --- JOG & CALIB TAB ---

f_ctl = ttk.LabelFrame(t_jog, text="Manual Controls"); f_ctl.pack(pady=10)

self.step = tk.IntVar(value=2)

tk.Scale(f_ctl, from_=1, to=10, var=self.step, orient='h').pack()

btns = [('W (Shld Up)',0,1,'w'), ('A (Base L)',1,0,'a'), ('S (Shld Dn)',1,1,'s'), ('D (Base R)',1,2,'d'),

('I (Elbow)',3,1,'i'), ('J (Extend)',4,0,'j'), ('K (Elbow)',4,1,'k'), ('L (Curl)',4,2,'l'),

('Open',5,0,'o'), ('Close',5,2,'p')]

bf = ttk.Frame(f_ctl); bf.pack(pady=10)

for t,r,c,k in btns: ttk.Button(bf, text=t, width=10, command=lambda k=k: self.jog(k)).grid(row=r, column=c)

f_cal = ttk.LabelFrame(t_jog, text="Calibration Profile (Select to Edit)"); f_cal.pack(pady=10, fill='x')

# --- PROFILE SWITCHER ---

self.edit_profile_var = tk.BooleanVar(value=True) # True=White, False=Black

row_prof = ttk.Frame(f_cal); row_prof.pack(pady=5)

ttk.Radiobutton(row_prof, text="Edit WHITE Profile", variable=self.edit_profile_var, value=True, command=self.reload_mapper_ui).pack(side="left", padx=10)

ttk.Radiobutton(row_prof, text="Edit BLACK Profile", variable=self.edit_profile_var, value=False, command=self.reload_mapper_ui).pack(side="left", padx=10)

ttk.Button(f_cal, text="1. Home", command=self.move_home).pack(fill='x')

for pt in ["a1", "h1", "h8", "a8"]:

ttk.Button(f_cal, text=f"Save {pt.upper()}", command=lambda p=pt: self.save_pt(p)).pack(fill='x')

ttk.Button(f_cal, text="Save Calibration File (For Selected Profile)", command=self.save_file).pack(fill='x')

# --- TEST TAB ---

f_tst = ttk.Frame(t_test); f_tst.pack(pady=10)

self.ent_sq = ttk.Entry(f_tst); self.ent_sq.pack()

ttk.Button(f_tst, text="Go To Square (Using Active Profile)", command=self.goto_sq).pack(pady=5)

ttk.Button(f_tst, text="Override (Pin XYZ)", command=self.pin_sq).pack(fill='x', pady=2)

ttk.Button(f_tst, text="Unpin (Delete Override)", command=self.unpin_sq).pack(fill='x')

ttk.Label(f_tst, text="-- SIMULATOR --").pack(pady=10)

self.ent_mv = ttk.Entry(f_tst); self.ent_mv.pack()

self.chk_cap = tk.BooleanVar()

ttk.Checkbutton(f_tst, text="Capture?", var=self.chk_cap).pack()

ttk.Button(f_tst, text="Execute Move", command=self.run_sim).pack()

# --- VISION GAME TAB ---

f_vis = ttk.LabelFrame(t_game, text="Vision"); f_vis.pack(pady=10, fill='x')

ttk.Button(f_vis, text="Start Camera", command=self.start_vision_stream).pack(fill='x')

f_grid = ttk.Frame(f_vis); f_grid.pack()

for i, pt in enumerate(['a8','h8','a1','h1']): ttk.Button(f_grid, text=pt.upper(), command=lambda p=pt: self.save_vis_pt(p)).grid(row=i//2, column=i%2)

self.calib_side_var = tk.BooleanVar(value=True)

ttk.Radiobutton(f_vis, text="Vision: White Side", variable=self.calib_side_var, value=True).pack()

ttk.Radiobutton(f_vis, text="Vision: Black Side", variable=self.calib_side_var, value=False).pack()

ttk.Button(f_vis, text="Save & Close", command=self.stop_vision_stream).pack(fill='x')

self.robot_color_var = tk.BooleanVar(value=True)

ttk.Label(t_game, text="Select Game Mode (Loads Matching Profile)", font=("Arial", 10, "bold")).pack(pady=10)

ttk.Radiobutton(t_game, text="Robot is WHITE (Moves First)", variable=self.robot_color_var, value=True).pack()

ttk.Radiobutton(t_game, text="Robot is BLACK (Waits)", variable=self.robot_color_var, value=False).pack()

ttk.Button(t_game, text="START GAME", command=self.start_game_thread).pack(fill='x', pady=10)

ttk.Button(t_game, text="STOP", command=self.stop_game_gracefully).pack(fill='x')

self.txt = tk.Text(self.root, height=10); self.txt.pack(fill='both')

def load_calib_ui(self):

self.mapper = BoardMapper(is_white_profile=self.edit_profile_var.get())

self.calib_pts = self.mapper.phys_cal.copy()

p = "WHITE" if self.edit_profile_var.get() else "BLACK"

self.log(f"Switched to {p} Profile.")

def reload_mapper_ui(self):

self.load_calib_ui()

def save_pt(self, n):

x, y, z = get_xy_from_angles(self.b, self.s, self.e, self.w)

self.calib_pts[n] = [x, y]

self.log(f"Saved {n} [{x:.1f},{y:.1f}]")

def save_file(self):

is_white = self.edit_profile_var.get()

fn = "calibration_white.json" if is_white else "calibration_black.json"

json.dump(self.calib_pts, open(fn, 'w'))

self.reload_mapper_ui()

self.log(f"Saved to {fn}")

def pin_sq(self):

sq = self.ent_sq.get().strip().lower()

x, y, z = get_xy_from_angles(self.b, self.s, self.e, self.w)

d = self.mapper.ovr.copy()

d[sq] = [x, y, z, self.b, self.s, self.e, self.w, "ANGLES"]

fn = self.mapper.ovr_file

json.dump(d, open(fn, 'w'))

self.reload_mapper_ui()

self.log(f"Pinned {sq} to {fn}")

def unpin_sq(self):

sq = self.ent_sq.get().strip().lower()

d = self.mapper.ovr.copy()

if sq in d:

del d[sq]

json.dump(d, open(self.mapper.ovr_file, 'w'))

self.reload_mapper_ui()

self.log(f"Unpinned {sq}")

def jog(self, k):

s = self.step.get()

if k=='w': self.s+=s;

if k=='s': self.s-=s

if k=='a': self.b+=s;

if k=='d': self.b-=s

if k=='i': self.e+=s;

if k=='k': self.e-=s

if k=='j': self.w-=s;

if k=='l': self.w+=s

if k=='o': self.g=GRIP_OPEN_ANGLE;

if k=='p': self.g=GRIP_CLOSED_ANGLE

self.robot.send_cmd(self.b, self.s, self.e, self.w, self.g)

def move_home(self):

self.b, self.s, self.e, self.w, self.g = 90, 90, 180, 18, GRIP_OPEN_ANGLE

self.robot.home()

def goto_sq(self):

sq = self.ent_sq.get().strip().lower()

if sq in self.mapper.ovr:

val = self.mapper.ovr[sq]

if len(val) > 5:

self.robot.send_cmd(val[3], val[4], val[5], val[6], GRIP_CLOSED_ANGLE)

self.b, self.s, self.e, self.w = val[3], val[4], val[5], val[6]; return

x, y, z = self.mapper.get_coords(sq)

self.robot.move_to_xyz(x, y, z=Z_TRAVEL); time.sleep(0.5)

self.robot.move_to_xyz(x, y, z=z)

angles = self.robot.inverse_kinematics(x, y, z_height=z)

if angles: self.b, self.s, self.e, self.w = angles

def start_vision_stream(self):

self.vis_running = True; self.vis_pts = {}; self.last_click = None

threading.Thread(target=self._vision_loop, daemon=True).start()

def _vision_loop(self):

cap = cv2.VideoCapture(CAMERA_INDEX); cv2.namedWindow("Vision Setup")

def mouse_cb(event, x, y, flags, param):

if event == cv2.EVENT_LBUTTONDOWN: self.last_click = [x, y]

cv2.setMouseCallback("Vision Setup", mouse_cb)

while self.vis_running:

ret, frame = cap.read()

if not ret: break

if self.last_click: cv2.drawMarker(frame, (self.last_click[0], self.last_click[1]), (0, 255, 255), cv2.MARKER_CROSS, 20, 2)

cv2.imshow("Vision Setup", frame); cv2.waitKey(1)

cap.release(); cv2.destroyAllWindows()

def save_vis_pt(self, sq):

if self.last_click: self.vis_pts[sq] = self.last_click; self.log(f"Saved {sq}")

def stop_vision_stream(self):

self.vis_running = False

try:

self.v_zones = interpolate_grid(self.vis_pts['a8'], self.vis_pts['h8'], self.vis_pts['h1'], self.vis_pts['a1'])

fn = "vision_white.json" if self.calib_side_var.get() else "vision_black.json"

with open(fn, 'w') as f: json.dump(self.v_zones, f)

with open("vision_grid.json", 'w') as f: json.dump(self.v_zones, f)

self.log(f"Vision Saved to {fn}")

except: self.log("Vision Error: Missing Points?")

def start_game_thread(self):

if hasattr(self, 'game_thread') and self.game_thread.is_alive(): return self.log("Game Running!")

self.stop_requested = False

self.game_thread = threading.Thread(target=run_vision_game, args=(self.robot, self.robot_color_var.get(), self))

self.game_thread.start()

def stop_game_gracefully(self): self.stop_requested = True; self.log("Stopping...")

def run_sim(self): threading.Thread(target=self._sim).start()

def _sim(self):

m = self.ent_mv.get().strip().lower(); cap = (self.chk_cap.get() == 1)

def smart_move(x, y, z, g):

angles = self.robot.inverse_kinematics(x, y, z_height=z)

if angles:

self.robot.send_cmd(angles[0], angles[1], angles[2], angles[3], g)

self.b, self.s, self.e, self.w, self.g = angles[0], angles[1], angles[2], angles[3], g

self.robot.gripper_state = g; time.sleep(0.8)

else: print(f"SIM ERR: {x:.1f},{y:.1f},{z:.1f}")

def land_on(sq, g):

if sq in self.mapper.ovr and len(self.mapper.ovr[sq]) > 5:

val = self.mapper.ovr[sq]

self.robot.send_cmd(val[3], val[4], val[5], val[6], g)

self.b, self.s, self.e, self.w, self.g = val[3], val[4], val[5], val[6], g; time.sleep(0.8); return

rx, ry, rz = self.mapper.get_coords(sq); smart_move(rx, ry, rz, g)

if len(m) < 4: return

x1, y1, _ = self.mapper.get_coords(m[0:2]); x2, y2, _ = self.mapper.get_coords(m[2:4])

self.robot.send_cmd(self.b, self.s, self.e, self.w, GRIP_OPEN_ANGLE); self.g = GRIP_OPEN_ANGLE

if cap:

print(f"Sim Capture {m[2:4]}")

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

land_on(m[2:4], GRIP_OPEN_ANGLE)

land_on(m[2:4], GRIP_CLOSED_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_CLOSED_ANGLE)

self.robot.send_cmd(180, 90, 90, 90, GRIP_CLOSED_ANGLE)

self.b, self.s, self.e, self.w = 180, 90, 90, 90; time.sleep(0.8)

self.robot.send_cmd(180, 90, 90, 90, GRIP_OPEN_ANGLE); time.sleep(0.5)

print(f"Sim Move {m}")

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)

land_on(m[0:2], GRIP_OPEN_ANGLE)

land_on(m[0:2], GRIP_CLOSED_ANGLE)

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_CLOSED_ANGLE)

land_on(m[2:4], GRIP_CLOSED_ANGLE)

land_on(m[2:4], GRIP_OPEN_ANGLE)

smart_move(x2, y2, Z_HOVER, GRIP_OPEN_ANGLE)

self.robot.home()

if __name__ == "__main__":

root = tk.Tk(); app = RobotGUI(root)

def on_closing(): root.destroy(); os._exit(0)

root.protocol("WM_DELETE_WINDOW", on_closing); root.mainloop()

Is Stop and Park in the latest version?

Yes. Both Stop and Park are included and fully integrated into the v8.6 code I provided.

They work together as a safety sequence:

1. The "STOP" Button

In the GUI (Tab 3), there is a STOP button:

Python

ttk.Button(t_game, text="STOP", command=self.stop_game_gracefully).pack(fill='x')

- What it does: It sets a "flag" (stop_requested = True).

- Response Time: The robot will finish its current physical movement (to avoid dropping a piece mid-air) and then immediately exit the game loop.

2. The "PARK" Function (Automatic)

The code is designed so that whenever the game ends (either because of Checkmate OR because you clicked STOP), the robot automatically executes the Park command.

You can see this at the very bottom of the run_vision_game function:

Python

# ... (Game Loop Ends) ...

cap.release(); cv2.destroyAllWindows(); engine.quit()

print("Game Stopped.");

robot.park()  # <--- THIS TRIGGERS AUTOMATICALLY

The Park Pose

The park() function (inside the Hardware Driver section) sends the robot to a specific "folded" safety position using raw hardware values:

Python

def park(self):

# Sends hardcoded safety angles to tuck the arm away

msg = f"1550,1400,1289,1289,{self.deg_to_us(GRIP_CLOSED_ANGLE)}\n"

self.ser.write(msg.encode())

Summary of Behavior

- You click STOP.

- Robot finishes the current specific move (e.g., placing a piece).

- Robot immediately stops thinking or looking for moves.

- Robot folds itself into the Park Position.

Is the Square Wave movement code incorporated in the simulation and game play also?

Yes, the "Square Wave" (Safety Lift) movement is fully incorporated in BOTH the Game Loop and the Simulation.

The code strictly enforces a "Up $\to$ Over $\to$ Down" logic to ensure the robot never drags a piece across the board and knocks over other pieces.

Here is exactly how the code handles it in both sections:

1. In Real Game Play (run_vision_game)

Every single move (Standard, Capture, or Castling) follows this strict Z_HOVER sequence:

Python

# 1. ELEVATOR UP (Square Wave Start)

# The robot moves to the Source X,Y but stays high at Z_HOVER (12cm)

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)

# 2. ELEVATOR DOWN

# It descends vertically to the exact square height to grab

land(sq_src, GRIP_OPEN_ANGLE)

# 3. GRAB & LIFT (Square Wave Middle)

raw_move(..., GRIP_CLOSED_ANGLE)       # Close Gripper

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE) # Lift piece straight up to 12cm

# 4. TRAVERSE HIGH

# It moves through the air at Z_HOVER to the destination

smart_move(x2, y2, Z_HOVER, GRIP_CLOSED_ANGLE)

# 5. ELEVATOR DOWN (Square Wave End)

land(sq_dst, GRIP_CLOSED_ANGLE)

2. In The Simulator (_sim)

I wrote the Simulator to mirror the real robot's behavior exactly, so you can visually verify the safety path.

Lines 470-480 in the code show the same logic:

Python

print(f"Sim Move {m}")

smart_move(x1, y1, Z_HOVER, GRIP_OPEN_ANGLE)   # Move to Source High

land_on(m[0:2], GRIP_OPEN_ANGLE)               # Drop Down

land_on(m[0:2], GRIP_CLOSED_ANGLE)             # Grip

smart_move(x1, y1, Z_HOVER, GRIP_CLOSED_ANGLE) # Lift Up High

smart_move(x2, y2, Z_HOVER, GRIP_CLOSED_ANGLE) # Travel High

land_on(m[2:4], GRIP_CLOSED_ANGLE)             # Drop Down

Why this matters

If the robot moved directly from A1 to D4 without the "Square Wave" (Lift), the piece hanging from the gripper would crash into pawns on row 2 or 3. By forcing Z_HOVER between every X/Y change, the robot creates a "bridge" over the other pieces.

This is a classic robotics problem called "Obstacle Clearance."

Currently, your robot is likely moving diagonally from Start (High) to End (Low). This diagonal path clips the tall pieces next to it.

You need to enforce a "Square Wave" Motion:

- Lift Up (Vertical)

- Travel Over (Horizontal at High Altitude)

- Drop Down (Vertical)

Here is how to modify both the Simulator and the Vision Game logic in chess_robot_monolith.py.

1. Define a Safe "Hover" Height

First, ensure you have a constant at the top of your file that is higher than your tallest piece (King/Queen).

Python

# Add/Check this at the top of your file

Z_HOVER = 12.0  # Safe travel height (cm). Increase if you have very tall pieces.

2. Fix the Vision Game Logic (move_piece)

Find your move_piece function (sometimes called execute_move). You must split the movement into distinct "Hover" and "Descend" steps.

Replace your movement logic with this structure:

Python

def move_piece(self, start_sq, end_sq):

# 1. Get Coordinates

x1, y1, z1 = self.mapper.get_coords(start_sq)

x2, y2, z2 = self.mapper.get_coords(end_sq)

# --- PHASE 1: PICK UP ---

# Go to Start X,Y but stay HIGH first

self.robot.move_to_xyz(x1, y1, Z_HOVER)

# Descend vertically to grab

self.robot.move_to_xyz(x1, y1, z1)

self.robot.grasp()

time.sleep(0.5)

# LIFT UP vertically (Crucial!)

self.robot.move_to_xyz(x1, y1, Z_HOVER)

# --- PHASE 2: TRAVEL ---

# Move Horizontally to Destination X,Y while keeping Z HIGH

# This prevents hitting neighbors during travel

self.robot.move_to_xyz(x2, y2, Z_HOVER)

# --- PHASE 3: PLACE ---

# Now that we are directly over the square, descend vertically

self.robot.move_to_xyz(x2, y2, z2)

self.robot.release()

time.sleep(0.5)

# Lift back up before going home (to avoid hitting the piece you just placed)

self.robot.move_to_xyz(x2, y2, Z_HOVER)

# Go Home

self.robot.move_to_home()

3. Fix the Simulator Logic (_sim)

Find your def _sim(self): function. It likely has the same diagonal movement issue.

Update the movement section:

Python

def _sim(self):

# ... (your existing code to get text input s1, s2) ...

x1, y1, z1 = self.mapper.get_coords(s1)

x2, y2, z2 = self.mapper.get_coords(s2)

# 1. Approach Start (High -> Low)

self.robot.move_to_xyz(x1, y1, Z_HOVER)  # Hover over Start

self.robot.move_to_xyz(x1, y1, z1)       # Descend

self.robot.grasp()

# 2. Lift (Vertical)

self.robot.move_to_xyz(x1, y1, Z_HOVER)  # Lift straight up

# 3. Travel (Horizontal)

self.robot.move_to_xyz(x2, y2, Z_HOVER)  # Fly over the board

# 4. Place (Vertical)

self.robot.move_to_xyz(x2, y2, z2)       # Descend

self.robot.release()

# 5. Clear (Vertical)

self.robot.move_to_xyz(x2, y2, Z_HOVER)  # Lift away

Summary of the Fix

- Old Way (Bad): move_to_xyz(x2, y2, z2) -> The robot tries to go from High Start to Low End in one go, cutting a diagonal line through your other pieces.

- New Way (Good): move_to_xyz(x2, y2, Z_HOVER) -> Forces the robot to fly over the crowd first, then drop down only when safe.

Next Step: Apply these changes to both functions and run a test with the King surrounded by Pawns. It should hop over them cleanly.