Chess Robot Arm (Vision + AI)

This project is a fully autonomous chess-playing robot arm. It uses Computer Vision (OpenCV) to see the board, Stockfish AI to calculate moves, and a 5-DOF Robotic Arm to physically move pieces.

The system features a custom Inverse Kinematics solver, a smooth-motion Arduino driver, and a Tkinter-based GUI for easy calibration.

🎥 Features

- Computer Vision: Detects human moves using a webcam and grid-based difference detection.

- Intelligent AI: Integrated with Stockfish 16 (or compatible UCI engine) for high-level gameplay.

- Inverse Kinematics: Custom Python solver handles coordinate-to-angle conversion with "tilt correction" for gravity compensation.

- Smooth Motion: Arduino firmware handles acceleration and deceleration to prevent piece toppling.

- Safety Parking: Automatic "tuck-in" mode ($90^\circ$ Base, $70^\circ$ Shoulder, $80^\circ$ Elbow) when idle or powered on.

🛠️ Hardware Requirements

- Robot Arm: 5-DOF or 6-DOF arm (Servos: MG996R or similar).

- Microcontroller: Arduino Uno/Nano (running robot_firmware.ino).

- Camera: Standard USB Webcam (mounted overhead).

- Chess Set: Standard tournament board (green/white squares recommended for contrast).

- Power Supply: 5V 3A+ external power supply for servos (Do not power via USB).

📂 Project Structure

Plaintext

├── arduino/

│   └── robot_firmware.ino       # The code running on the Arduino

├── chess_robot_monolith.py      # Main application (GUI, Game Loop, Vision)

├── arm.py                       # Python driver & Inverse Kinematics logic

├── stockfish/                   # Folder containing the Stockfish engine executable

├── calibration.json             # (Auto-generated) Saved board coordinates

└── vision_config.json           # (Auto-generated) Saved camera zones

🚀 Installation

1. Python Dependencies

Install the required libraries:

Bash

pip install opencv-python numpy pyserial python-chess

2. Stockfish Setup

- Download Stockfish from stockfishchess.org.

- Place the executable file inside a folder named stockfish/ or update the path in chess_robot_monolith.py:

Python

STOCKFISH_PATH = r"path/to/your/stockfish.exe"

3. Arduino Setup

- Open arduino/robot_firmware.ino.

- Verify pin assignments (Default: Base=10, Shoulder=8, Elbow=9, Wrist=12, Gripper=11).

- Upload to your board.

- Note: The robot will immediately move to the "Parking Position" upon power-up.

🎮 How to Run

- Connect the Arduino via USB.

- Connect the Webcam.

- Run the main script:

Bash

python chess_robot_monolith.py

The Workflow

- Vision Calibration: Click "Run Vision Calibration" in the GUI. Adjust the grid corners until they match your physical board.

- Motor Calibration: (Optional) Use the sliders to fine-tune the (0,0) coordinate if the arm is slightly off-center.

- Start Game: Click "Start Vision Game".

- Playing:

  - You (White): Make a move physically. Remove your hand.

  - Confirm: Press SPACEBAR on the keyboard.

  - Robot (Black): The arm will pick up and move its piece automatically.

⚠️ Safety & Parking

The robot uses a specific "Parking Position" to save space and protect servos.

- Base: $90^\circ$ (Forward)

- Shoulder: $70^\circ$ (Leaning Forward)

- Elbow/Wrist: $80^\circ$ (Curled In)

- Gripper: $34^\circ$ (Open)

Always ensure the area in front of the robot is clear before powering on.

🔧 Troubleshooting

- "Shoulder Heavy" Warning: The target is too far. Move the board closer to the robot base.

- Camera lagging: Ensure lighting is consistent. Shadows can trigger false move detections.

- Jittery movement: Check your external power supply amperage.

⚙️ Adjustable Parameters & Calibration

This robot relies on several key parameters to translate chess moves into physical servo angles. These can be adjusted directly in the Python code or tweaked via the GUI during calibration.

1. Physical Dimensions (arm.py)

These constants define the geometry of your specific robot arm. Measure your arm segments in cm and update these values in arm.py if they differ.

- L_BASE_H: Height from the table surface to the shoulder joint axis.

- L_SHOULDER: Length from shoulder pivot to elbow pivot.

- L_ELBOW: Length from elbow pivot to wrist pivot.

- L_WRIST: Length from wrist pivot to the tip of the gripper fingers.

2. Tuning Offsets

Hardware isn't perfect. Gravity causes sag, and servos may not be perfectly centered.

- OFFSET_SHOULDER: Adds a few degrees to the shoulder to counteract the weight of the extended arm (gravity compensation). Increase this if the arm consistently undershoots (hits the board).

- OFFSET_BASE: Rotational offset if the robot isn't perfectly aligned with the board's centerline.

3. Height & Tilt Strategy

The code uses a "Tilt Correction" algorithm to keep the gripper parallel to the table even when the arm stretches out.

- Z_TRAVEL: The safe "hover" height (cm) for moving pieces across the board.

- Z_GRIP: The height (cm) at which the gripper closes on a piece.

- TILT_SLOPE & TILT_INTERCEPT: Linear equation parameters that adjust the Z-height based on distance from the base. This compensates for the natural "droop" of the arm at full extension.

4. Servo Limits (Pulse Widths)

In arduino/robot_firmware.ino (and mirrored in Python), we map degrees (0-180) to Pulse Width Modulation (PWM) signals (microseconds).

- MIN_PULSE (Default: 400): The pulse width for 0 degrees.

- MAX_PULSE (Default: 2400): The pulse width for 180 degrees.

- Note: If your servos are buzzing or getting hot at the extremes, adjust these values to a safer range (e.g., 500-2300).

5. Calibration Process (GUI)

The chess_robot_monolith.py script includes a graphical interface to make calibration easy without coding.

- Launch GUI: Run python chess_robot_monolith.py.

- Vision Calibration: Click "Run Vision Calibration". Click the 4 corners of the chessboard on the live video feed. This maps camera pixels to board squares.

- Coordinate Testing: Enter a square name (e.g., "e4") in the manual control box.

- Offset Sliders: If the robot misses the center of the square, use the X/Y/Z offset sliders in the GUI to nudge it. These offsets are temporary for the session; update arm.py if permanent changes are needed.

📄 License

This project is open-source. Feel free to modify and improve!